Experiment 4

Evaluate Low-Power CMOS-Inspired RTL Architectures Using Switching-Activity Reduction

Aim

To compare a conventional 8-bit adder register with a low-switching version that uses operand isolation and an enable control, and to verify through behavioral simulation how the low-power design reduces unnecessary switching activity.

Software Used

Xilinx Vivado Design Suite 2024.1 (XSim simulator).

Conventional Design

module conventional_power_design (

input  wire       clk,

input  wire       rst,

input  wire [7:0] a,

input  wire [7:0] b,

output reg  [7:0] y

);

always @(posedge clk or posedge rst) begin

if (rst)

y <= 8'b0;

else

y <= a + b;

end

endmodule

Low-Switching Design

Operand isolation is included so that the arithmetic inputs become zero when computation is disabled.

module low_power_design (

input  wire       clk,

input  wire       rst,

input  wire       enable,

input  wire [7:0] a,

input  wire [7:0] b,

output reg  [7:0] y

);

wire [7:0] a_iso;

wire [7:0] b_iso;

wire [7:0] sum;

assign a_iso = enable ? a : 8'b0;

assign b_iso = enable ? b : 8'b0;

assign sum = a_iso + b_iso;

always @(posedge clk or posedge rst) begin

if (rst)

y <= 8'b0;

else if (enable)

y <= sum;

end

endmodule

Testbench Code

`timescale 1ns/1ps

module tb_low_power;

reg clk;

reg rst;

reg enable;

reg [7:0] a,b;

wire [7:0] y_conv;

wire [7:0] y_low;

conventional_power_design D1 (

.clk(clk),

.rst(rst),

.a(a),

.b(b),

.y(y_conv)

);

low_power_design D2 (

.clk(clk),

.rst(rst),

.enable(enable),

.a(a),

.b(b),

.y(y_low)

);

always #5 clk = ~clk;

initial begin

clk=0;

rst=1;

enable=0;

a=0;

b=0;

#10 rst=0;

enable=1;

a=10; b=20;

#10;

a=15; b=25;

#10;

enable=0;

a=50; b=60;

#10;

a=100; b=100;

#10;

enable=1;

a=30; b=40;

#10;

$finish;

end

endmodule

Output

Results at each rising clock edge (values in decimal, hexadecimal in brackets):

| Clock edge (ns) | enable | a | b | y_conv | y_low |
| --- | --- | --- | --- | --- | --- |
| 5 (reset) | 0 | 0 | 0 | 0 | 0 |
| 15 | 1 | 10 | 20 | 30 (1E) | 30 (1E) |
| 25 | 1 | 15 | 25 | 40 (28) | 40 (28) |
| 35 | 0 | 50 | 60 | 110 (6E) | 40 (28) held |
| 45 | 0 | 100 | 100 | 200 (C8) | 40 (28) held |
| 55 | 1 | 30 | 40 | 70 (46) | 70 (46) |

Simulation waveform (Vivado behavioral simulation):

Fig. 1: Behavioral simulation of tb_low_power (simulation time 60 ns)

Observation

While enable = 1, both designs give the same output (30, 40 and 70). While enable = 0 (30 ns to 50 ns), the conventional design keeps adding the changing inputs, so y_conv changes to 110 and then 200. In the low-power design the adder inputs are forced to zero and the output register is not updated, so y_low holds its previous value of 40. This removes the unnecessary switching in the adder and register, which lowers dynamic power. Simulation shows the reduced activity only; the actual power saving would be measured with Vivado power analysis.

Result

The conventional and low-switching designs were simulated. The low-power design gives the same results as the conventional design when enabled and holds its output with no input-driven switching when disabled, which verifies the switching-activity reduction.