Experiment 3

Evaluate Logical Effort and Multistage Delay Optimization for Large Capacitive Loads

Aim

To implement a 3-input AND function as single-stage, two-stage and three-stage Verilog designs, verify their functionality through behavioral simulation, and study how splitting the logic into multiple stages relates to logical effort and delay optimization.

Software Used

Xilinx Vivado Design Suite 2024.1 (XSim simulator).

Single-Stage Design

module single_stage (

input wire a,

input wire b,

input wire c,

output wire y

);

assign y = a & b & c;

endmodule

Two-Stage Design

module two_stage (

input wire a,

input wire b,

input wire c,

output wire y

);

(* keep = "true" *) wire s1;

assign s1 = a & b;

assign y  = s1 & c;

endmodule

Three-Stage Design

module three_stage (

input wire a,

input wire b,

input wire c,

output wire y

);

(* keep = "true" *) wire s1;

(* keep = "true" *) wire s2;

assign s1 = a & b;

assign s2 = s1 & c;

assign y  = s2;

endmodule

Testbench Code

`timescale 1ns/1ps

module tb_multistage;

reg a,b,c;

wire y1,y2,y3;

single_stage A1(a,b,c,y1);

two_stage    A2(a,b,c,y2);

three_stage  A3(a,b,c,y3);

initial begin

$monitor("T=%0t A=%b B=%b C=%b Y1=%b Y2=%b Y3=%b",

$time,a,b,c,y1,y2,y3);

a=0;b=0;c=0;

#10 a=1;

#10 b=1;

#10 c=1;

#10 a=0;

#10 b=0;

#10 c=0;

#10 $finish;

end

endmodule

Output

Console output of the testbench ($monitor):

T=0 A=0 B=0 C=0 Y1=0 Y2=0 Y3=0

T=10 A=1 B=0 C=0 Y1=0 Y2=0 Y3=0

T=20 A=1 B=1 C=0 Y1=0 Y2=0 Y3=0

T=30 A=1 B=1 C=1 Y1=1 Y2=1 Y3=1

T=40 A=0 B=1 C=1 Y1=0 Y2=0 Y3=0

T=50 A=0 B=0 C=1 Y1=0 Y2=0 Y3=0

T=60 A=0 B=0 C=0 Y1=0 Y2=0 Y3=0

Input and output levels obtained from simulation:

| Time (ns) | A | B | C | Y1 | Y2 | Y3 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 – 10 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 – 20 | 1 | 0 | 0 | 0 | 0 | 0 |
| 20 – 30 | 1 | 1 | 0 | 0 | 0 | 0 |
| 30 – 40 | 1 | 1 | 1 | 1 | 1 | 1 |
| 40 – 50 | 0 | 1 | 1 | 0 | 0 | 0 |
| 50 – 60 | 0 | 0 | 1 | 0 | 0 | 0 |
| 60 – 70 | 0 | 0 | 0 | 0 | 0 | 0 |

Simulation waveform (Vivado behavioral simulation):

Fig. 1: Behavioral simulation of tb_multistage (simulation time 70 ns)

Observation

The outputs Y1, Y2 and Y3 of the single-stage, two-stage and three-stage designs are identical. Each goes high only when A, B and C are all 1 (30 ns to 40 ns) and is low otherwise. In behavioral simulation the stages add no delay, so splitting the logic changes only its structure, not its function. The delay and logical-effort benefit of multistage design, especially for large capacitive loads, is seen in synthesis and timing analysis of the implemented design.

Result

The single-stage, two-stage and three-stage implementations of y = a & b & c were simulated. All three give the same logical output for every input combination, which verifies the functional correctness of the designs.